Chapter 2, Problem 59.

Three light bulbs are connected in series to a 120-V source as shown in Fig. 2.123.  Find the current I through the bulbs.  Each bulb is rated at 120 volts.  How much power is each bulb absorbing?  Do they generate much light?

##### 120 V

Figure 2.123

For Prob. 2.59.