Chapter 2, Solution 23

In the circuit shown in Fig. 2.87, determine vx and the power absorbed by the 12- resistor.

##### 20 A

Figure 2.87

For Prob. 2.23.

Solution

8//12 = 4.8,  3//6 = 2,  (4 + 2)//(1.2 + 4.8) = 6//6 = 3

The circuit is reduced to that shown below.

ix  	1

+     vx      –

20A

Applying current division,

ix = [2/(2+1+3)]20 = 6.667 A and vx = 1x6.667 = 6.667 V

The current through the 1.2  resistor is 0.5ix = 3.333 A.  The voltage across the 12  resistor is 3.333 x 4.8 = 16V.  Hence the power absorbed by the 12 Ω resistor is equal to

(16)2/12 = 21.33 W