### Chapter 2, Solution 83

Two delicate devices are rated as shown in Fig. 2.142.  Find the values of the resistors R1 and R2 needed to power the devices using a 24-V battery.

Figure 2.142

For Prob. 2.83.

Solution

The voltage across the fuse should be negligible when compared with 24 V (this can be checked later when we check to see if the fuse rating is exceeded in the final circuit).  We can calculate the current through the devices.

I1 =

I2 =

# R1

-

+

# 24 V

# R2

# i1 = 5 mA

# i2 = 20 mA

# iR1

Ifuse

# iR2

Let R3 represent the resistance of the first device, we can solve for its value from knowing the voltage across it and the current through it.

R3 = 9/0.005 = 1,800 Ω

This is an interesting problem in that it essentially has two unknowns, R1 and R2 but only one condition that need to be met and that the voltage across R3 must equal 9 volts.  Since the circuit is powered by a battery we could choose the value of R2 which draws the least current, R2 = ∞.  Thus, we can calculate the value of R1 that give 9 volts across R3.

9 = (24/(R1 + 1800))1800 or R1 = (24/9)1800 – 1800 = 3 kΩ

This value of R1 means that we only have a total of 25 mA flowing out of the battery through the fuse which means it will not open and produces a voltage drop across it of 0.05V.  This is indeed negligible when compared with the 24-volt source.