# Chapter 3, Solution 68

Although there are many ways to work this problem, this is an example based on the same kind of problem asked in the third edition.

Problem

Find the voltage Vo in the circuit of  Fig. 3.112.

40

20

24 V

+

_

4A

Vo

25

10

3 A

+

_

Figure 3.112

For Prob. 3.68.

# Solution

Consider the circuit below.  There are two non-reference nodes.

40

20

24 V

+

_

4 A

Vo

25

10

3 A

+

_

# V1	Vo

Using MATLAB, we get,

>> Y=[0.125,-0.1;-0.1,0.19]

Y =

0.1250   -0.1000

-0.1000    0.1900

>> I=[7,-2.04]'

I =

7.0000

-2.0400

>> V=inv(Y)*I

V =

81.8909

32.3636

Thus, Vo = 32.36 V.

We can perform a simple check at node Vo,

3 + 0.1(32.36–81.89) + 0.05(32.36) + 0.04(32.36–24) =

3 – 4.953 + 1.618 + 0.3344 = – 0.0004; answer checks!