Chapter 4, Solution 33.

### 10

### 5

#### 10

##### 4 A

Determine the Thevenin equivalent circuit as seen by the 5-ohm resistor.  Then calculate the current flowing through the 5-ohm resistor.

Solution

Step 1.		We need to find Voc and Isc.  To do this, we will need two circuits, label the appropriate unknowns and solve for Voc, Isc, and then Req which is equal to Voc/Isc.

### 10

#### 10

##### 4 A

### 10

#### 10

##### 4 A

##### V2

##### V1

##### +

##### Voc

##### –

##### Isc

Note, in the first case V1 = Voc and the nodal equation at 1 produces –4+(V1–0)/10 = 0.  In the second case, Isc = (V2–0)/10 where the nodal equation at 2 produces,

–4+[(V2–0)/10]+[(V2–0)/10] = 0.

Step 2.		0.1V1 = 4 or V1 = 40 V = Voc = VThev.  Next, (0.1+0.1)V2 = 4 or 0.2V2 = 4 or V2 = 20 V.  Thus, Isc = 20/10 = 2 A.  This leads to Req = 40/2 = 20 Ω.  We can check our results by using source transformation.  The 4-amp current source in parallel with the 10-ohm resistor can be replaced by a 40-volt voltage source in series with a 10-ohm resistor which in turn is in series with the other 10-ohm resistor yielding the same Thevenin equivalent circuit.  Once the 5-ohm resistor is connected to the Thevenin equivalent circuit, we now have 40 V across 25 Ω producing a current of 1.6 A.