# Chapter 4, Solution 56.

Use Norton’s theorem to find Vo in the circuit of Fig. 4.122.

+

_

10 k

36 V

+

_

Vo

12 k

2 k

24 k

3 mA

1 k

Figure 4.122

For Prob. 4.56.

Solution

We remove the 1-k resistor temporarily and find Norton equivalent across its terminals.  Req is obtained from the circuit below.

10 k

RN

12 k

2 k

24 k

Req =  10 + 2 + (12//24) = 12+8 = 20 k

IN is obtained from the circuit below.

+

_

10 k

36 V

12 k

2 k

24 k

3 mA

IN

We can use superposition theorem to find IN.  Let   IN = I1 + I2, where I1 and I2 are due to 16-V and 3-mA sources respectively.  We find I1 using the circuit below.

+

_

10 k

36 V

12 k

2 k

24 k

I1

Using source transformation, we obtain the circuit below.

12 k

12 k

I1

24 k

3 mA

12//24 = 8 k

To find I2, consider the circuit below.

10 k

12 k

2 k

24 k

3 mA

I2

2k + 12k//24 k   = 10 k

I2=0.5(-3mA) = -1.5 mA

IN = 1.2 –1.5 = -0.3 mA

20 k

1 k

In

a

b

The Norton equivalent with the 1-k resistor is shown below

+

Vo

–

Vo = 1k(20/(20+1))(-0.3 mA) = -285.7 mV.