Chapter 4, Solution 72.

- For the circuit in Fig. 4.138, obtain the Thevenin equivalent at terminals a-b.

- Calculate the current in RL = 8.

- Find RL for maximum power deliverable to RL.

- Determine that maximum power.

Figure 4.138

For Prob. 4.72.

Solution

(a)	RTh and VTh are calculated using the circuits shown in Fig. (a) and (b) respectively.

From Fig. (a),		RTh  =  2 + 4 + 6  =  12 ohms

#### 6

### 2

#### (a)

##### RTh

#### 4

#### 20V

##### +

##### VTh

##### 8V

+

+

+

#### 2

#### 6

#### (b)

#### 4

#### 12V

From Fig. (b), 		-VTh + 12 + 8 + 20  =  0,  or  VTh  =  40 V

(b)			i  =  VTh/(RTh + R)  =  40/(12 + 8)  =  2A

(c)		For maximum power transfer,		RL  =  RTh  =  12 ohms

(d)		p  = VTh2/(4RTh)  =  (40)2/(4x12)  =  33.33 watts.