Chapter 5, Solution 90.

The op amp circuit in Fig. 5.107 is a current amplifier.  Find the current gain io/is of the amplifier.

Figure 5.107

For Prob. 5.90.

Solution

Transforming the current source to a voltage source produces the circuit below,

### 4 k

##### 5is

+

#### 20 k

##### +

##### vo

+

#### 5 k

### 2 k

##### a

##### b

##### io

At node b,		vb  =  (2/(2 + 4))vo  =  vo/3

At node a,		(5is – va)/5  =  (va – vo)/20

But va = vb = vo/3.	20is – (4/3)vo  =  (1/3)vo – vo, or is  =  vo/30

io  =  [(2/(2 + 4))/2]vo  =  vo/6

io/is  =  (vo/6)/(vo/30)  =  5