Chapter 5, Solution 92

Refer to the bridge amplifier shown in Fig. 5.109. Determine the voltage gain vo/vi .

Figure 5.109

For Prob. 5.92.

Solution

The top op amp circuit is a non-inverter, while the lower one is an inverter.   The output at the top op amp is

v1  =  (1 + 60/30)vi  =  3vi

while the output of the lower op amp is

v2  =  -(50/20)vi  =  -2.5vi

Hence,		vo  =  v1 – v2  =  3vi + 2.5vi  =  5.5vi

vo/vi  =  5.5