Chapter 6, Problem 68.

A 10–V dc voltage is applied to an integrator with R = 50 k, C = 100 F at t = 0.  How long will it take for the op amp to saturate if the saturation voltages are +12 V and –12 V?  Assume that the initial capacitor voltage was zero.