Chapter 6, Solution 34.

The current through a 10-mH inductor is 10e-t/2 A.  Find the voltage and the power at t = 3 s.

Solution

i = 10e–t/2

= –50e-t/2 mV

v(3) = –50e–3/2 mV =  –11.157 mV

p = vi = –500e–t mW

p(3) = –500e–3 mW = –24.89 mW.