# Chapter 6, Solution 48.

Under steady-state dc conditions, find i and v in the circuit in Fig. 6.71.

+

v

### 30 k

#### i

2 mH

### 20 k

### 6 µF

### 5 mA

Figure 6.71

For Prob. 6.48.

Solution

Under steady-state, the inductor acts like a short-circuit, while the capacitor acts like an open circuit as shown below.

i

+

5 mA	30k        	  v               20 k

–

Using current division,

i = (30k/(30k+20k))(5mA) = 3 mA

v = 20ki  = 60 V