# Chapter 7, Solution 7.

Assume that the switch in Fig. 7.87 has been in position A for a long time and is moved to position B at t=0.  Then at t = 1second, the switch moves from B to C.  Find vC(t) for t  0.

10 k

2 mF

12V

+

500

1 k

A

B

C

Figure 7.87

For Prob. 7.7

Solution

Step 1.		Determine the initial voltage on the capacitor.  Clearly it charges to 12 volts when the switch is at position A because the circuit has reached steady state.

This then leaves us with two simple circuits, the first a 500 Ω resistor in series with a 2 mF capacitor and an initial charge on the capacitor of 12 volts.  The second circuit which exists from t = 1 sec to infinity.  The initial condition for the second circuit will be vC(1) from the first circuit.  The time constant for the first circuit is (500)(0.002) = 1 sec and the time constant for the second circuit is (1,000)(0.002) = 2 sec.  vC(∞) = 0 for both circuits.

Step 1.

vC(t) = 12e-t volts for 0 < t < 1 sec and = 12e-1e-2(t-1) at t = 1 sec, and

= 4.415e-2(t-1) volts for 1 sec < t < ∞.

12e-t volts for 0 < t < 1 sec, 4.415e-2(t-1) volts for 1 sec < t < ∞.