Chapter 7, Solution 64

#### Req

### 10 H

### iL(t)

##### vThev(t)

+

Determine the value of iL(t) and the total energy dissipated by the circuit from

#### 40

### 10 H

### 40

### iL(t)

##### vin(t)

+

##### v1

t = 0 sec to t = ∞ sec.  The value of vin(t) is equal to [40–40u(t)] volts.

Solution

Step 1.		Determine the Thevenin equivalent circuit to the left of the inductor.  This means we need to find voc(t) and isc(t) which gives us vThev(t) = voc(t) and Req = voc(t)/isc(t) (note, this only works for resistor networks in the time domain).  This leads to the second circuit shown above.

Now, with this circuit, we can use the generalized solution to a first order differential equation, iL(t) = Ae–(t–0)/τ+B where, t0 = 0, τ = L/R, A+B = iL(0) and 0+B = iL(∞).  Finally, we can use w = (1/2)LiL(t)2 to calculate the energy dissipated by the circuit (w = [(1/2)LiL(∞)2–(1/2)LiL(0)2].

Step 2.		We now determine the Thevenin equivalent circuit.  First we need to pick a reference node and mark the unknown voltages, as seen above.  With the inductor out of the circuit, the node equation is simply [(v1–vin(t))/40] + [(v1–0)/40] + 0 (since the inductor is out of the circuit, there is an open circuit where it was) = 0.  This leads to [(1/40)+(1/40)]v1 = (1/40)vin(t) or 2v1 = vin(t) or v1 = 0.5vin(t) = [20–20u(t)] voc(t) = vThev(t).  Now to short the open circuit which produces v1 = 0 and isc = –[(0–vin(t))/40] = vin(t)/40 = 0.025vin(t) A.

Step 3.		Now, everything comes together, Req = voc(t)/isc(t) = 0.5vin(t)/[0.025vin(t)] = 0.5/0.025 = 20 Ω.  Next we find τ = L/Req = 10/20 = (1/2) sec.  At t = 0–, vin(0–) = [10–0] V (note u(t) = 0 until t = 0).  Since it has been at this value for a very long time, the inductor can be considered a short and the value of the current is equal to 20/20 or iL(0–) = 1 amp.  Since you cannot change the current instantaneously, iL(0) = 1 amp = A+B.  Since vThev(t) = 20–20 = 0 for all t > 0, all the energy in the inductor will be dissipated by the circuit and iL(∞) = 0 = B which means that A = 1 and iL(t) = [e–2t] u(t) amps.  The total energy dissipated from t = 0 to ∞ sec is equal to [(1/2)LiL(0)2–(1/2)LiL(∞)2] = (0.5)10(1)2–0 = 5 J.