Chapter 8, Solution 34.

Calculate i(t) for t > 0 in the circuit in Fig. 8.82.

50u(–t) V

### 5

Figure 8.82

For Prob. 8.34.

Solution

Before t = 0, the capacitor acts like an open circuit while the inductor behaves like a short circuit.

i(0) = 0, v(0) = 50 V

For t > 0, the LC circuit is disconnected from the voltage source as shown below.

#### (¼) H

#### (1/16)F

#### i

+

#### Vx

This is a lossless, source-free, series RLC circuit.

- =  R/(2L)  =  0, o = 1/ =  1/ =  8,  s =  j8

Since  is equal to zero, we have an undamped response.  Therefore,

i(t) = A1cos(8t) + A2sin(8t)  where i(0) = 0 = A1

di(0)/dt = (1/L)vL(0) = –(1/L)v(0) = –4x50 = –200

However, di/dt = 8A2cos(8t), thus, di(0)/dt = –200 = 8A2 which leads to A2 = –25

Now we have 			i(t)  =  –25sin(8t) A