Chapter 8, Solution 47.

Find the output voltage vo(t) in the circuit of Fig. 8.94.

Figure 8.94

For Prob. 8.47.

Solution

At  t  =  0-,  we obtain,	iL(0)  =  3x5/(10 + 5)  =  1A

and  vo(0)  =  0.

For  t  >  0,  the 10-ohm resistor is short-circuited and we have a parallel RLC circuit with a step input.

- =  1/(2RC)  =  (1)/(2x5x0.01)  =  10

o = 1/ =  1/ =  10

Since   =  o, we have a critically damped response.

s1,2  =    -10

Thus,			i(t)  =  Is + [(A + Bt)e-10t],   Is  =  3

i(0)  =  1  =  3 + A  or  A  =  -2

vo  =  Ldi/dt  =   [Be-10t] + [-10(A + Bt)e-10t]

vo(0)  =  0  =  B – 10A  or  B  =  -20

Thus,	vo(t)  =   (200te-10t) V