Chapter 8, Solution 59.

The switch in Fig. 8.105 has been in position 1 for t < 0. At t =0, it is moved from position 1 to the top of the capacitor at t = 0. Please note that the switch is a make before break switch, it stays in contact with position 1 until it makes contact with the top of the capacitor and then breaks the contact at position 1.  Determine v(t).

Figure 8.105

For Prob. 8.59.

Solution

Let  i = inductor current and  v = capacitor voltage

v(0) = 0,  i(0) = 40/(4+16) = 2A

For t>0, the circuit becomes a source-free series RLC with

i(0) = 2 = A

v = –32te–2t V.

Checking,

v = Ldi/dt + Ri = 4(–4e–2t – 4e–2t + 8e–2t) + 16(2e–2t – 4te–2t) = –32te–2t V.