Chapter 10, Solution 76.

Determine Vo and Io in the op amp circuit of Fig. 10.119.

230° V

Figure 10.119

For Prob. 10.76.

Solution

Let the voltage between the -jk  capacitor and the 10k resistor be V1.

(1)

= 1.7321+j1

Also,

(2)

Solving (2) into (1) yields

= (4+j5)Vo

= 312.4–21.34˚ mV

Io = (V1–Vo)/20k = Vo/(–j4k) = (0.3124/4k)(–21.43+90)˚

= 78.168.57˚ µA

We can easily check this answer using MATLAM.  Using equations (1) and (2) we can identify the following matrix equations:

YV = I where

>> Y=[1-0.6i,0.6i;1,-1-0.5i]

Y =

1.0000 - 0.6000i        0 + 0.6000i

1.0000            -1.0000 - 5.0000i

>> I=[1.7321+1i;0]

I =

1.7321 + 1.0000i

0

>> V=inv(Y)*I

V =

0.8593 + 1.3410i

0.2909 - 0.1137i  = Vo = 312.3–21.35˚ mV.  The answer checks.