Chapter 10, Solution 85.

Using Fig. 10.127, design a problem to help other students to better understand performing AC analysis with PSpice.

Although there are many ways to solve this problem, this is an example based on the same kind of problem asked in the third edition.

Problem

Use PSpice to find Vo in the circuit of Fig. 10.127.  Let R1 = 2 Ω, R2 = 1 Ω, R3 = 1 Ω, R4 = 2 Ω, Is = 20˚ A, XL = 1 Ω, and XC = 1 Ω.

# Solution

The schematic is shown below.   We let rad/s so that L=1H and C=1F.

When the circuit is saved and simulated, we obtain from the output file

FREQ			VM(1)		VP(1)

1.591E-01		2.228E+00	-1.675E+02

From this, we conclude that

Vo = 2.228–167.5° V.