# Chapter 11, Solution 39.

An ac motor with impedance  ZL = 4.2  + j3.6 Ω is supplied by a 220-V, 60-Hz source.  (a)  Find pf, P, and Q. (b)  Determine the capacitor required to be connected in parallel with the motor so that the power factor is corrected to unity.

Solution

  - ZL = 4.2  + j3.6  = 5.5317 40.6o

pf = cos 40.6  = 0.7592

## P = 6.643 kW

## Q = 5.695  kVAR

(b)  , 312 µF

{It is important to note that this capacitor will see a peak voltage of 220 = 311.08V, this means that the specifications on the capacitor must be at least this or greater!}