# Chapter 12, Solution 8.

In a balanced three-phase wye-wye system, the source is an abc-sequence of voltages and Van = 10020˚ V rms.  The line impedance per phase is (0.6+j1.2), while the per phase impedance of the load is (10+j14) .  Calculate the line currents and the load voltages.

Solution

Consider the per phase equivalent circuit shown below.

Zl

+

_

Van

ZL

5.396–35.1˚ A

Ia = Van/( Zl + ZL) = (100∠20°)/(10.6+j15.2) = (100∠20°)/(18.531∠55.11°)

= 5.396∠–35.11° amps.

Ib = Ia ∠–120° = 5.396∠–155.11° amps.

Ic = Ia ∠+120° = 5.396∠84.89° amps.

VLa = IaZL = (4.414–j3.103)(10+j14) =  (5.396∠–35.11°)(17.205∠54.46°)

= 92.84∠19.35° volts.

VLb = VLa ∠–120° = 94.84∠–100.65° volts.

VLc = VLa ∠+120° = 94.84∠139.35° volts.