Chapter 13, Solution 56.

Find the power absorbed by the 10- resistor in the ideal transformer circuit of Fig. 13.120.

Figure 13.121

For Prob. 13.56.

Solution

We apply mesh analysis to the circuit as shown below.

#### 1 : 2

##### 46V

+

#### I1

#### I2

##### +

##### v2

#### 10

#### 5

#### 2

##### +

##### v1

For mesh 1,			46  =  7I1 – 5I2 + v1				(1)

For mesh 2,			v2  =  15I2 – 5I1  				(2)

At the terminals of the transformer,

v2  =  nv1  =  2v1				(3)

I1  =  nI2  =  2I2				(4)

Substituting (3) and (4) into (1) and (2),

46  =  9I2 + v1					(5)

v1  =  2.5I2					(6)

Combining (5) and (6),	46  =  11.5I2  or  I2  =  4

P10  =  0.5|I2|2(10)  =  80 watts.