Chapter 14, Solution 33.

A parallel resonant circuit with quality factor 120 has a resonant frequency of 6x106 rad/s.  Calculate the bandwidth and half-power frequencies.

Solution

B = ωo/Q = 6x106/120 = 50 krad/s.

ω1 = ωo – B/2 = 5.975x106 rad/s and ω2 = ωo + B/2 = 6.025x106 rad/s.

50 krad/s, 5.975x106 rad/s, 6.025x106 rad/s