Chapter 16, Solution 2.

The differential equation that describes the voltage in an RLC network is

d2vdt2+5dvdt+4v=0

Given that v(0) = 0, dv(0)/dt = 5 V/s, obtain v(t).

Solution

The differential equation that describes the voltage in an RLC network is

Given that v(0) = 0, dv(0)/dt = 5, obtain v(t).

Solution

Step 1.		Transform the equation into the s-domain and solve for V(s).

s2V(s) – (dv(0–)/dt) – sv(0–) + 5sV(s) – 5v(0–) +4V(s) = 0 or

(s2 + 5s +4)V(s) – 5 = 0 or V(s) = 5/(s2+5s+4)

Step 2.		Perform a partial fraction expansion of V(s) and then solve for v(t).

s2 + 5s + 4  =  0,  thus  s1,2  =    =  –4,  –1.

Thus, V(s) = 5/[(s+1)(s+4)] = A/(s+1) + B/(s+4) where A = 5/3 and B = –5/3

Thus,

v(t)  =  [(5/3)e–t – (5/3)e–4t]u(t) V