Chapter 16, Solution 8.

A branch voltage in an RLC circuit is described by

If the initial conditions are v(0) = 0 = dv(0)/dt, find v(t).

Solution

Step 1.	First, we transform the equation into the s-domain.  Then we solve for V(s).

s2V(s) – (dv(0–)/dt) – sv(0–) + 4sV(s) – 4v(0–) + 8V(s) = 48/s or

s2V(s) + 4sV(s) + 8V(s) = 48/s = (s2+5s+8)V(s) or

V(s) = 48/[s(s2+4s+8)]

Step 2.	Now we need to solve for the roots of the denominator and perform a partial fraction expansion.  Then we can inverse transform the answer back into the time domain.

s2 + 4s + 8 has the roots s1,2  =  thus,

V(s) = 48/[s(s+2+j2)(s+2–j2)] = [A/s] + [B/(s+2+j2)] + [C/(s+2–j2)]

where A = 48/4 = 6; B = 48/[(–2–j2)(–j4)] = 48/(–8+j8) =

48(–1–j)/[8(–1+j)( –1–j)] = 6(–1–j)/2 = 3(–1–j); and C =

48/[(–2+j2)(j4)] = 48/(–8–j8) = 48(–1+j)/[8(–1–j)(–1+j)] = 6(–1+j)/2 =

3(–1+j).

Therefore, V(s) = [8/s] + [3(–1–j)/(s+2+j2)] + [3(–1+j)/(s+2–j2)]

v(t) = [6 – 6e-2t(cos2t + sin2t)]u(t) volts