Chapter 16, Solution 21.

In the circuit of Fig. 16.44, the switch moves (make before break switch) from position A to B at t = 0. Find v(t) for all t  0.

Figure 16.44

For Prob. 16.21.

Solution

Step 1.		First we need to find our initial conditions, clearly i(0) = 0 and v(0) = 4x15 = 60 volts.  Next we convert the circuit into the s-domain.  We can then write a mesh equation and solve for v(t).

### I

### 10

#### s/4

### 25/s

##### 60/s

+

[10+(s/4)+(25/s)]I + 60/s = 0 and V = (25/s)I + 60/s

Step 2.		I = –(60/s)/[ 10+(s/4)+(25/s)] = –(60/s){4s/[s2+40s+100]

= –240/[(s+2.679)(s+37.32)] = [A/(s+2.679)]+[B/(s+37.32)] where

A = –240/(–2.679+37.32) =  –6.928 and B = –240/(–37.32+2.679) = 6.928.

This now leads to V = (25/s)I+60/s

= {(25)(–6.928)/[s(s+2.679)]}+{(25)(6.928)/[s(s+37.32)]}+60/s

= {–173.2/[s(s+2.679)]}+{173.2/[s(s+37.32)]}+60/s

= [a/s]+[b/(s+2.679)]+[c/(s+37.32)] where

a = [–173.2/2.679]+[173.2/37.32]+60 = –64.65+4.641+60 = –0.009 (In practice and theoretically, this term must be equal to be zero since there will be no energy in the circuit at t = ∞!);

b = [–173.2/(–2.679)] = 64.65; and c = 173.2/(–37.32) = –4.641 or –4.65 if we correct the rounding errors.  Thus,

v(t) = [64.65e–2.679t–4.65e–37.32t]u(t) volts.