Chapter 16, Solution 22.

Find the voltage across the capacitor as a function of time for t > 0 for the circuit in Fig. 16.45.  Assume steady-state conditions exist at t = 0-.

Figure 16.45

For Prob. 16.22.

Solution

Step 1.		First we need to calculate the initial conditions, vC(0) = 0 and iL(0) = 20/5 = 4 amps.  Next, we need to convert the circuit into the s-domain and solve for the node voltage V = VC.  Convert this back into the time domain and obtain vC(t).

### V

### 1

### 1/s

### 4/s

### s/4

[(V–0)/1]+[4/s]+[(V–0)/(s/4)]+[(V–0)/(1/s)] = 0 then solve for V, next complete a partial fraction expansion, and then convert back into the time domain.

Step 2.		[1+(4/s)+s]V = [(s2+s+4)/s]V = –4/s or

V = –4/[(s+0.5+j1.9365)(s+0.5–j1.9365)]

= [A/(s+0.5+j1.9365)] + [B/(s+0.5–j1.9365)] where A = –4/(–j3.873)

= 1.0328–90˚ and B = –4/(j3.873) = 1.032890˚.  Thus,

vC(t) = 1.0328e–t/2[e–(j1.9365t+90˚)+e(j1.9365t+90˚)]u(t)

= 2.066e–t/2cos(1.9365t+90˚)u(t) volts.