Chapter 8, Solution 24.

The switch in the circuit of Fig. 16.47 has been closed for a long time but is opened at t = 0.  Determine i(t) for t > 0.

Figure 16.47

For Prob. 16.24.

8.20

Solution

#### 2

#### 0.5s

#### 6/s

### 4/s

+

##### 12/s

### I

Step 1.		First we solve for the initial conditions and then convert the circuit into the s-domain and then solve for I, perform a partial fraction expansion, then convert back into the time domain.  We recognize that the capacitor becomes an open circuit and the inductor becomes a short circuit at t = 0–.  Therefore, v(0) = 12 volts and i(0) = 12/2 = 6 amps.

We can use mesh analysis, –(12/s) + (4/s)I + (0.5s)(I–6/s) + 2I = 0.

Step 2.		[(4/s)+0.5s+2]I = (12/s)+(3) = (3s+12)/s = [(s2+4s+8)/(2s)]I or

I = (6s+24)/[(s+2+j2)(s+2–j2)] = [A/(s+2+j2)] + [B/(s+2–j2)] where

A = (–12–j12+24)/(–j4) = 16.97–45˚/4–90˚= 4.24345˚and

B = (–12+j12+24)/(j4) = 4.243–45˚.  Thus,

i(t) = 4.243e–2t[e–(j2t–45˚)+e(j2t–45˚)]u(t)

= 8.486e–2tcos(2t–45˚) amps.