Chapter 16, Problem 26.

##### i(t)

##### B

##### A

### 12 A

### 20

### 10 mF

##### t = 0

### 10

### 0.25H

The switch in Fig. 16.49 moves from position A to position B at t=0 (please note that the switch must connect to point B before it breaks the connection at A, a make before break switch).  Determine i(t) for t >0.  Also assume that the initial voltage on the capacitor is zero.

Figure 16.49

For Prob. 16.26.

Solution

##### V

##### I

### 12/s

### 100/s

### 10

### 0.25s

Step 1.		Determine the initial conditions and then convert the circuit into the s-domain.  Then solve for V and then find I.  Convert it into the time domain.  It is clear from the circuit that iL(0) = 12 A.

Applying nodal analysis we get,

[(V–0)/(100/s)]+[(V–0)/10]+[(V–0)/(0.25s)]+12/s = 0 where

I = [(V–0)/(0.25s)]+12/s.

Step 2.		[(s2+10s+400)/(100s)]V = –12/s or

V = –1200/[(s+5+j19.365)(s+5–j19.365)]  or

I = –{4800/[s(s+5+j19.365)(s+5–j19.365)]}+12/s

= [A/s]+[B/(s+5+j19.365)]+[C/(s+5–j19.365)] where A = –12+12 = 0, as to be expected; B = –4800/[(–5–j19.365)(–j38.73)]

= 4800180˚/[(20–104.48)(38.73–90˚)] = 6.19714.48˚; and

C = –4800/[(–5+j19.365)(j38.73)] = 4800180˚/[(20104.48)(38.7390˚)]

= 6.197–14.48˚.

i(t) = [12.394e–5tcos(19.365t+14.48˚)]u(t) amps.