Chapter 16, Problem 27.

Find v(t) for t > 0 in the circuit in Fig. 16.50.

Figure 16.50

For Problem 16.27.

Solution

Step 1.		First we need to determine the initial conditions.  We note that the source on the right is equal to zero until the switch opens.  So, all initial conditions come from the 3-amp source on the left.  Since the capacitor looks like an open and the inductor looks like a short we get,

v(0) = 3[5x10/(5+10)] = 10 volts and iL(0) = 10/5 = 2 amps.

#### 5

##### 20/s

+

#### s

#### 2/s

##### +

##### V

### 1/(4s)

##### 10/s

+

#### I

Next we convert the circuit (t > 0) into the s-domain with initial conditions.  Then we can solve for V, perform a partial fraction expansion and solve for v(t).

–[10/s]+[1/(4s)]I+[s(I–(2/s))]+5I+[20/s] = 0 and V = [1/(4s)](–I) + [10/s]

Step 2.		{[1/(4s)]+s+5}I = {[s2+5s+0.25]/(s)}I = 2–10/s = 2(s–5)/s or

I = 2(s–5)/[(s+0.05051)(s+4.949)] and

V = {–2(s–5)/[(4s)(s+0.05051)(s+4.949)]}+10/s

= {–0.5(s–5)/[s(s+0.05051)(s+4.949)]}+10/s

V = [A/s] + [B/(s+0.05051)] + [C/(s+4.949)] where

A = [2.5/[(0.05051)(4.949)]]+10 = 20;

B = –0.5(–0.05051–5)/[(–0.05051)(–0.05051+4.949)]

= 2.52525/(–0.24742) = –10.206; and

C = –0.5(–4.949–5)/[(–4.949)(–4.949+0.05051)] = 4.9745/24.243 = 0.2052.

v(t) = [20–10.206e–0.05051t+0.2052e–4.949t]u(t) volts.

dv/dt = –10.206(–0.05051)+0.2052(–4.949) = 0.5155–1.0155 = –0.5 or

Cdv/dt = –4x0.5 = –2 amps, the answer checks!