Chapter 16, Solution 31.

Obtain v(t) and i(t) for t > 0 in the circuit in Fig. 16.54.

Figure 16.54

For Prob. 16.31.

Solution

Step 1.		First, find the initial conditions and then transform the above circuit into the s-domain after converting the current source in parallel with the 5-ohm resistor into a 15 volts voltage source in series with a 5-ohm resistor.  Then solve for V and I, perform a partial fraction expansion on each and then convert back into the time domain.  The steady state the values are i(0) = 0 and v(0) = 20 volts.

##### 15/s

+

#### 1

#### 5

#### 2

+

#### 20/s

#### 5s

##### +

##### V

##### 20/s

+

### 5/s

### I

–[15/s]+5I+(5s)I+1I+[5/s]I+[20/s]–[20/s]+2I = 0 and V = {[5/s]I+[20/s].

Step 2.		{5+[5s]+1+[5/s]+2}I = [15/s]–[20/s]+[20/s] = 15/s or

{5[s2+1.6s+1]/s}I = 15/s or I = 3/[(s+0.8+j0.6)(s+0.8–j0.6)] and

V = {[5/s]I+[20/s] = 15/[s(s+0.8+j0.6)(s+0.8–j0.6)] + 20/s

= [A/s] + [B/(s+0.8+j0.6)] + [C/(s+0.8–j0.6)] where A = [15/(0.64+0.36)]+20 = 35; B = 15/[(–0.8–j0.6)(–j1.2)] = 12.590˚/1–143.13˚ = 12.5–126.87˚; and

C = 15/[(–0.8+j0.6)(j1.2)] = 12.5–90˚/(1143.13˚ = 12.5126.87˚.

v(t) = [35+12.5e–0.8t–j0.6t–126.87˚+12. 5e–0.8t+j0.6t+126.87˚]u(t) volts

= [35+25e–0.8tcos(0.6t+126.87˚)]u(t) volts.

I = 3/[(s+0.8+j0.6)(s+0.8–j0.6)] = [A/(s+0.8+j0.6)] + [B/(s+0.8–j0.6)] where

A = 3/(–j1.2) = 2.590˚ and B = 3/(j1.2) = 2.5–90˚.  Thus,

i(t) = 2.5e–0.8t[e–j0.6t+90˚+ej0.6t–90˚]u(t)

= 5e–0.8t[cos(0.6t–90˚)]u(t) amps.