Chapter 16, Solution 32.

For the network in Fig. 16.55, solve for i(t) for t > 0.

Figure 16.55

For Prob. 16.32.

Solution

Step 1.		First, we need to find all the initial conditions.  Then we need to transform the circuit into the s-domain and solve for I.  We then perform a partial fraction expansion and convert the results into the time domain.  The inductor becomes a short and the capacitor becomes an open circuit.  Thus, i(0) = [20/6] + [20/12] = 5 amps and vC(0) = 10 + 10 = 20 volts.

##### 20/s

+

### 8/s

##### 5/s

### 0.5s

##### 10/s

+

### 4

#### I

+

Loop equation, –[10/s]–0.5s(I–5/s)–4I–[8/s]I+[20/s] = 0.

Step 2.		[0.5s+4+8/s]I = [(s2+8s+16)/(2s)]I = –[10/s]+2.5+20/s = (s+4)/(0.4s) or

I = 5(s+4)/[(s+4)2] = [A/(s+4)]+[B/(s+4)2] where A(s+4)+B = 5(s+4) or A = 5 and B = 0.  Therefore,

i(t) = [5e–4t]u(t) amps.