Chapter 16, Solution 37.

Determine v(t) for t > 0 in the circuit in Fig. 16.60.

#### 30

### 20

#### 0.25H

##### 30u(t)V

+

##### 60u(t)V

+

##### +  v

#### 0.5F

Figure 16.60

For Prob. 16.37.

Solution

#### 30

### 20

#### s/4

##### 30/s

+

##### 60/s

+

##### +  V

#### 2/s

Step 1.		First, we need to determine the initial conditions for this circuit.  Since both sources were zero (shorts) until t = 0, the initial conditions for this circuit are equal to zero (v(0) = 0 and iL(0) = 0).  Next, we transform the circuit into the s-domain.  Then we can write node equations and then solve for V.  Then perform a partial fraction expansion and convert back into the time domain.

From this circuit there are different ways of solving for v(t).  Perhaps the easiest is to replace the circuit seen by the capacitor and inductor with a Thevenin equivalent circuit.  VThev = [(60/s)/(30+20)]20 – 30/s = (24/s)–(30/s) = –6/s and Req = 20x30/(20+30) = 12 Ω.  Thus, we now have the following circuit where we can now find I.  Once we have I we can find V and then perform a partial fraction expansion and then convert into the time domain to solve for v(t).

##### +  V

#### 12

#### s/4

##### –6/s

+

#### 2/s

#### I

–[–6/s] + 12I + [2/s]I + [s/4]I = 0 and V = [2/s]I.

Step 2.		[(s/4)+12+(2/s)]I = –6/s = [(s2+48s+8)/(4s)]I or

I = (–6/s)(4s)/ [(s2+48s+8)] = –24/[(s+0.165)(s+47.84)] and

V = –48/[s(s+0.1672)(s+47.84)] = [A/s]+[B/(s+0.1672)]+[C/(s+47.84)] where

A = –48/[0.1672x47.84] = 6; B = –48/[–0.1672(–0.1672+47.84)] = 6.022; and

C = –48/[–47.84(–47.84+0.1672)] = –0.021

Therefore,

v(t) = [–6+6.022e–0.1672t–0.021e–47.84t]u(t) volts.