Chapter 16, Solution 43.

Determine i(t) for t > 0 in the circuit of Fig. 16.66.

Figure 16.66

For Prob. 16.43.

Solution

##### I

### 5

### 20/s

##### 3/s

##### V

### 5s

##### 6/s

Step 1.		First, we need to determine the initial conditions.  Then we need to transform the circuit into the s-domain.  Once in the s-domain we can calculate V and I.  We then perform a partial fraction expansion on I and convert back into the time domain.  Since the inductor looks like a short just before the switch opens, vC(0) = 0 and i(0) = (12/4)+3 = 6 amps.

[(V–0)/(5s)] + [6/s] + [(V–0)/(20/s)] + [(V–0)/5] – [3/s] = 0 and I = [(V–0)/(5s)] + [6/s].

Step 2.		[(1/(5s))+(s/20)+(1/5)]V = [(s2+4s+4)/(20s)]V = –3/s or

V = –60/(s+2)2 and I = –[12/(s(s+2)2)] + 6/s = [A/s] + [B/(s+2)] + [C/(s+2)2] + (6/s) where A = –3 and A(s2+4s+4) + B(s2+2s) + Cs = –12

= –3s2 – 12s –12 + Bs2 + B(2s) + Cs or –3 B = 0 or B = 3 and

–12 + 6 + C = 0 or C = 6.

i(t) = [3+3e–2t+6te–2t]u(t) amps.