Chapter 16, Solution 44.

For the circuit in Fig. 16.67, find i(t) for t > 0.

##### 120 V

##### 24u(t) A

Figure 16.67

For Prob. 16.44.

Solution

Step 1.		First we identify the initial conditions.  Then we simplify the circuit (for t > 0) and then transform it into the s-domain.  We simplify the circuit by performing a source transformation and combining the 10 Ω resistor with the 40 Ω resistor (10x40/(10+40) = 8 Ω) and adding the two current sources together 12+24 = 36 A.  We then solve for the node voltage, V, and then find I.  Finally we perform a partial fraction expansion and convert the answer into the time domain.  For t < 0, the inductor looks like a short circuit producing vC(0) = 0 and i(0) = 120/10 = 12 amps.

##### I

### 8

##### 36/s

##### V

### 4s

##### 12/s

### 100/s

–[36/s] + [(V–0)/(100/s)] + [(V–0)/8] + [(V–0)/(4s)] + [12/s] = 0 and

I = [(V–0)/(4s)] + [12/s].

Step 2.		[(s/100)+(1/8)+1/(4s)]V = [(s2+12.5s+25)/(100s)]V = 24/s or

V = 2,400/[(s+2.5)(s+10)] and I = 600/[s(s+2.5)(s+10)]+[12/s]

= [A/s] + [B/(s+2.5)] + [C/(s+10)] where A = 24+12 = 36;

B = 600/[–2.5(–2.5+10)]  = –32; and C = 600/[–10(–10+2.5)] = 8.

i(t) = [36–32e–2.5t+8e–10t]u(t) amps.