Chapter 16, Solution 50.

For the circuit in Fig. 16.73, find v(t) for t > 0.  Assume that v(0+) = 4 V and

i(0+) = 2 A.

Figure 16.73

For Prob. 16.50.

Solution

Step 1.		Determine the initial condition of the second capacitor and then convert the circuit into the s-domain.  Finally, solve for V, perform a partial fraction expansion and convert the answer back into the time domain.  Since v(0) = 4 volts and i(0) = 2 amps then –4–2(2)+v2(0) = or v2(0) = 8.

I/4

2

+

V

–

4/s

+

10/s

8/s

+

2/s

I

[(V–4/s)/(10/s)]–[I/4]+[(V–8/s)/(2+2/s)] = 0 and I = [((8/s)–V)/(2+2/s)]

= [4/(s+1)]–0.5sV/(s+1)

Step 2.		[(V–4/s)/(10/s)]+[0.125sV/(s+1)]–[1/(s+1)]+[(V–8/s)/(2+2/s)]

[(s/10)+(0.125s/(s+1))+(0.5s/(s+1))]V =

[0.4+4/(s+1)]+[1/(s+1)] = (0.4s+5.4)/(s+1)

= [(s2+s+6.25s)/(10(s+1))]V = [s(s+7.25)/(10(s+1))]V or

V = 4(s+13.5)/[s(s+7.25)] = [A/s]+[B/(s+7.25)] where A = 4(13.5)/7.25 = 7.748 and B = 4(–7.25+13.5)/(–7.25) = –3.448 or

v(t) = [7.748–3.448e–7.25t]u(t) volts.