Chapter 16, Solution 55.

Obtain i1 and i2 for t > 0 in the circuit of Fig. 16.78.

Figure 16.78

For Prob. 16.55.

Solution

#### 3

Step 1.		The first thing we do is to determine the initial conditions.  Since there is no excitation of the circuit before t = 0, all initial conditions must be zero.  Next we convert the circuit into the s-domain.  Then use nodal analysis and eventually solve for I1 and I2, then perform a partial fraction expansion and convert back into the time domain.

##### V1

##### I1

##### I2

### s

### 2

##### 4/s

### s

–[4/s]+[(V1–0)/2]+[(V1–0)/s]+[(V1–0)/(s+3)] = 0 and I1 = [(V1–0)/s] and

I2 = [(V1–0)/(s+3)].

Step 2.		{[1/2]+[1/s]+[1/(s+3)]}V1 = 4/s = {[s2+3s+2s+6+2s]/[2s(s+3)]}V1 or

V1 = 8(s+3)/[s2+7s+6] = 8(s+3)/[(s+1)(s+6)] and I1 = 8(s+3)/[s(s+1)(s+6)]

= [A/s]+[B/(s+1)]+[C/(s+6)] where A = 8x3/6 = 4; B = 8(–1+3)/[(–1)(–1+6)]

= –16/5 = –3.2; C = 8(–6+3)/[(–6)(–6+1)] = –24/30 = –0.8.  Thus,

i1(t) = [4–3.2e–t–0.8e–6t]u(t) amps.

I2 = [(V1–0)/(s+3)] = 8/[(s+1)(s+6)] = [A/(s+1)] + [B/(s+6)] where A = 8/5 = 1.6 and B = 8/(–6+1) = –1.6.  Thus,

i2(t) = [1.6e–t–1.6e–6t]u(t) amps.

[4–3.2e–t–0.8e–6t]u(t) amps, [1.6e–t–1.6e–6t]u(t) amps