# Monday, December 10, 2018

# CHAPTER 8

# P.P.8.1

- At t = 0–, we have the equivalent circuit shown in Figure (a).

#### 50mF

#### 10

### 2

#### (a)

##### +

##### v

##### 24V

+

### 2

#### (b)

##### +

##### vC

##### 24V

+

#### vL

+

#### a

#### i

#### i

i(0–)  =  24/(2 + 10)  =  2 A,  v(0–)  =  2i(0–)  =  4 V

hence,  v(0+)  =  v(0–)  =  4 V.

- At  t  =  0+, the switch is closed.

L(di/dt)  =  vL,  leads to (di/dt)  =  vL/L

But,	vC(0+) + vL(0+)  =  24  =  4 + vL(0+), or vL(0+)  =  20 V

(di(0+)/dt)  =  20/0.4  =  50 A/s

C(dv/dt)  =  iC  leading to (dv/dt)  =  iC/C

But at node a,  KCL gives  i(0+)  =  iC(0+) + v(0+)/2  =  2  =  iC(0+) + 4/2

or  iC(0+)  =  0,  hence (dv(0+)/dt)  =  0 V/s

(c)	As t approaches infinity, the capacitor is replaced by an open circuit and the inductor is replaced by a short circuit.

v()  =  24 V,  and  i()  =  12 A.

P.P.8.2

(a)	At  t  =  0-,  we have the equivalent circuit shown in (a).

#### 5

#### (a)

#### iL

#### iR

#### 6A

#### 4A

##### +

##### vL

#### 2H

#### 10 F

#### 5

#### (b)

#### 6A

##### +

##### vC

+

#### vR

#### b

#### a

iL(0-)  =  –6A,  vL(0–)  =  0,  vR(0–)  =  0

At  t  =  0+,  we have the equivalent circuit in Figure (b).  At node b,

iR(0+)  =  iL(0+) + 6,  since iL(0+)  =  iL(0–)  =  –6A,  iR(0+)  =  0,

and vR(0+)  =  5iR(0+)  =  0.  Thus,  iL(0)  =  –6 A,  vC(0)  =  0,  and vR(0+)  =  0.

(b)	dvC(0+)/dt  =  iC(0+)/C  =  4/0.2  =  20 V/s.

To get (dvR/dt),  we apply KCL to node b,  iR  =  iL + 6,  thus  diR/dt  =  diL/dt.

Since  vR  =  5iR,  dvR/dt  =  5diR/dt  =  5diL/dt.  But  LdiL/dt  =  vL,  diL/dt  =  vL/L.

Hence,  dvR(0+)/dt  =  5vL(0+)/L.

Applying KVL to the middle mesh in Figure (b),

–vC(0+) + vR(0+) + vL(0+)  =  0  =  0 + 0 + vR(0+), or vR(0+)  =  0

Hence,  dvR(0+)/dt  =  0  =  diL(0+)/dt;

diL(0+)/dt  =  0,  dvC(0+)/dt  =  20 V/s,  dvR(0+)/dt  =  0.

(c)	As t approaches infinity, we have the equivalent circuit shown below.

#### 4A

#### 5

#### iL

#### 6A

- =  6 + iL()  leads to iL()  =  –2 A

vC()  = vR()  =  4x5  =  20 V

Thus,  iL()  =  –2 A,  vC()  =  vR()  =  20 V

P.P.8.3

=  R/(2L)  =  10/(2x5)  =  1,  o  =    =  10

s1,2  =     =  –1    j9.95.

Since   <  o,  we clearly have an underdamped response.

P.P.8.4		For t < 0, the  inductor is connected to the voltage source and when the circuit reaches steady state, the inductor acts like a short circuit.

i(0-)  =  50/10  =  5  =  i(0+)  =  i(0)

The voltage across the capacitor is  0  =  v(0-)  =  v(0+)  =  v(0).

For t > 0, we have a source-free RLC circuit.

3

=  R/(2L)  =  5/(2x1)  =  2.5

Since  < o, we have an underdamped case.

s1,2  =    =  –2.5  j1.6583

i(t)  =  e-2.5t[A1cos1.6583t  +  A2sin1.6583t]

We now determine A1  and  A2.

i(0)  =  10  =  A1

di/dt  =  –2.5{e–2.5t[A1cos(1.6583t)  +  A2sin(1.6583t)]}

+ 1.6583e–2.5t[–A1sin(1.6583t)  +  A2cos(1.6583t)]

di(0)/dt  =  –(1/L)[Ri(0) + v(0)]  =  –2.5A1 + 1.6583A2

= –1[5x10+0] = –1[50]  =  –2.5(10) + 1.6583A2

A2  =  –15.076

Thus,  i(t)  = e–2.5t(10 cos 1.6583t –  15.076 sin 1.6583t) A

P.P.8.5			  =  1/(2RC)  =  1/(2x2x25x10-3)  =  10

10

since    =  o,  we have a critically damped response.  Therefore,

v(t)  =  [(A1 + A2t)e–10t]

v(0)  =  0  =  A1 + A2x0  =  A1,  which leads to v(t)  =  [A2te–10t].

dv(0)/dt  =  –(v(0) + Ri(0))/(RC)  =  –2x0.05/(2x25x10-3)  =  –2

dv/dt  =  [(A2 – 10A2t)e–10t]

At t  =  0,	–2  =  A2  therefore,  v(t)  =  (–2t)e–10t u(t) V

P.P.8.6		For t < 0, the switch is closed.  The inductor acts like a short circuit while the capacitor acts like an open circuit.  Hence,

i(0)  =  3A  and  v(0)  =  0.

- =  1/(2RC)  =  1/(2x20x4x10-3)  =  6.25

=  5

Since   >  o,  this is an overdamped response.

s1,2  =    =  –2.5 and –10

Thus,   v(t)  =  A1e-2.5t  + A2e-10t

v(0)  =  0  =  A1 + A2,  which leads to A2  =  –A1

dv(0)/dt  =  –(v(0) + Ri(0))/(RC)  =  –(20x4.5)12.5  =  –1125

But,  dv/dt  = –2.5A1e-2.5t  –10A2e-10t

At t  =  0,   –1125  =  –2.5A1  – 10A2  =  7.5A1  since  A1  =  –A2

A1   =  –150,    A2  =  150

Thus,    v(t)  =  150(e–10t  – e–2.5t)  V

P.P.8.7		The initial capacitor voltage is obtained when the switch is in position a.

v(0)  =  [2/(2 + 1)]18  =  12 V

The initial inductor current is   i(0)  =  0.

When the switch is in position b,  we have the RLC circuit with the voltage source.

- =  R/(2L)  =  10/(2x2.5)  =  2

=  4

Since    <  o,  we have an underdamped case.

s1,2  =    =  -2  j 3.464

Thus,   v(t)  =  vf  + [(A1cos3.464t + A2sin3.464t)e-2t]

where vf  =  v()  =  15,  the final capacitor voltage.  We now impose the initial conditions to get  A1  and   A2.

v(0)  =  12  =  15 + A1  leads to A1  =  –3

The initial capacitor current is the same as the initial inductor current.

i(0)  =  C(dv(0)/dt)  =  0  therefore,  dv(0)/dt  =  0

But,   dv/dt  =  3.464[{–A1sin(3.464t) + A2cos(3.464t)}e-2t]

–2[{A1cos(3.464t) + A2sin(3.464t)}e–2t]

dv(0)/dt  =  0 – 2A1 + 3.464A2,  which leads to A2  =  –6/3.464  =  –1.7321

Thus,    v(t)  =  15– (1.7321 sin 3.464t + 3 cos 3.464t)e-2t V

i  =  C(dv/dt),  vR  =  Ri  =  RC(dv/dt)  =  (1/4)dv/dt

=  (1/4)[(6 – 6)cos(3.464t) + (2x1.7321 + 3x3.464)sin(3.464t)]e–2t

vR(t)  = 3.464e–2t sin3.464t V

P.P.8.8 	When t  <  0,  v(0)  =  0,  i(0)  =  0;  for t  >  0,

=  0.25

i(t)  =  is  +  A1cost + A2sint  =  10 + A1cos(0.25t) + A2sin(0.25t)

i(0)  =  0  =  10 + A1,  therefore  A1  =  –10

Ldi(0)/dt  =  v(0)  =  0

But   di/dt  =  –A1 0.25sin(0.25t) + A2 0.25cos(0.25t)

At t  =  0,  di(0)/dt  =  0  =  0 + 0.25A2  leading to  i(t)  =  10(1 – cos(0.25t)) A

v(t)  =  Ldi/dt  =  20x10x0.25sint  =  50 sin(0.25t) V

P.P.8.9		At  t  =  0, the switch is open so that v(0)  =  0,  i(0–)  =  0		(1)

For  t  >  0, the switch is closed.  We have the equivalent circuit as in Figure (a).

### 10

#### (a)

#### 3A

##### i

#### 2 H

#### (1/20)FF

### 4

##### iC

##### +

##### v

### 10

#### (b)

#### 3A

##### i

### 4

##### iC

v(0+)  =  0,  i(0+)  =  0				(2)

–3 + iC + i  =  0				(3)

From (3), i(0+)  =  0  means that iC(0+)  =  3,  but iC(0+)  =  Cdv(0+)/dt

which leads to dv(0+)/dt  =  iC(0+)/C  =  3/(1/20)  =  60 V/s

As t approaches infinity, we have the equivalent circuit in (b).

i()  =  3 A,  v()  =  4i()  =  12V				(5)

Next we find the network response by turning off the current source as shown in

### 10

#### (c)

##### i

#### 2 H

#### (1/20)FF

### 4

##### iC

##### +

##### v

#### i

Figure (c).

Applying KVL gives		–v – 10iC + 4i + 2di/dt  =  0			(6)

Applying KCL to the top node,	i – iC  =  0

Namely,			i  =  iC  =  –Cdv/dt  =  –(1/20)dv/dt		(7)

Combining (6) and (7),	–v – (10/20)dv/dt – (4/20)dv/dt – (2/20)d2v/dt2  =  0.

or     (d2v/dt2) + 7(dv/dt) + 10  =  0

The characteristic equation is 	  s2 + 7s + 10  =  0  =  (s + 2)(s + 5)

This means that vn  =  (Ae-2t + Be-5t) and vf  =   v()  =  12 V.

Thus,  v  =  vf + vn     		v  =  12 + (Ae-2t + Be-5t)			(8)

v(0)  =  0  =  12 + A +B,  or  A + B  =  –12	(9)

dv/dt  =  (–2Ae-2t   – 5Be-5t)

dv(0)/dt  =  60  =  –2A – 5B

2A + 5B  =  –60				(10)

From (9) and (10),  A  =  0  and   B  =  –12.

Thus,  v(t)  =  12(1 – e-5t) V  for all t > 0.

But, from (3),  i  =  3 – iC  =  3 –(1/20)dv/dt  =  3 –(1/20)(60)e-5t

i(t)  =  3(1 – e-5t) A  for all t > 0.

P.P.8.10		For  t <0,  5u(t)  =  0  so that  v1(0-)  =  v2(0-)  =  0		(1)

For  t  >  0,  the circuit is as shown in Figure (a).

#### 1

#### (a)

##### v1

#### (1/3)F

##### 20V

+

#### 1

#### 0.5F

##### v2

##### +

##### v2

#### 1

#### (b)

##### 20V

+

#### 1

##### +

##### v1

i1  =  C1dv1/dt,  or  dv1/dt  =  i1/C1;  likewise  dv2/dt  =  i2/C2

i2(0+)  =  (v1(0+) – v2(0+))/1  =  (0 – 0)/1  =  0

(20 – v1(0+))/1  =  i1(0+) + i2(0+),  or  20  =  i1(0+)

Hence,			dv1(0+)/dt  =  20/(1/2)  =  40 V/s			(2a)

dv2(0+)/dt  =  0						(2b)

As t approaches infinity, the capacitors can be replaced by open circuits as shown in Figure (b).  Thus,

v1()  =  v2()  =  20V				(3)

Next we obtain the network response by considering the circuit in Figure (c).

#### 1

#### (c)

##### v1

#### (1/3)F

#### 1

#### 0.5F

##### v2

Applying KCL at node 1 gives   (v1/1) + (1/2)(dv1/dt) + (v1 – v2)/1  =  0

or				v2  =  2v1  +  (1/2)dv1/dt			(4)

Applying KCL at node 2 gives  (v1 – v2)/1  =  (1/3)dv2/dt

or  v1  =  v2 + (1/3)dv2/dt			(5)

Substituting (5) into (4) yields,

v2  =  2v2 +(2/3)(dv2/dt) + (1/2)(dv2/dt) + (1/6)d2v2/dt2

or,    (d2v2/dt2) + (7dv2/dt) + 6v2  =  0

Now we have,   s2 + 7s + 6  =  0  =  (s + 1)(s + 6)

Thus,   v2n  =  (Ae-t + Be-6t)  and  v2f  =  v2()  =  20V.

v2  =  v2n + v2f  =  20 + (Ae-t + Be-6t)

v2(0)  =  0  which implies that A + B  =  –20		(6)

dv2/dt  =  (–Ae-t – 6Be-6t)

dv2(0)  =  0  =  –A – 6B				(7)

From (6) and (7),   A  =  –24  and  B  =  4.

Thus,   v2(t)  =  (20 – 24e-t + 4e-6t) V

From (5),			v1  =  v2 + (1/3)dv2/dt

Thus,  v1(t)  =  (20 – 16e-t – 4e-6t) V

Now we can find,		vo = v1 – v2 =  8(e–t – e–6t) V, t > 0

P.P.8.11		Let v1 equal the voltage at non-inverting terminal of the op amp.  Then vo is equal to the output of the op amp.

At the non-inverting terminal,  (vs – vo)/R1  =  C1dv1/dt			(1)

At the output terminal of the op amp,   (v1 – vo)/R2  =  C2dvo/dt		(2)

We now eliminate v1 from (2),  v1  =  vo + R2C2dvo/dt			(3)

From (1)			vs  =  v1 + R1C1dv1/dt				(4)

Substituting (3) into (4) gives

vs  =  vo + R2C2dvo/dt + R1C1dvo/dt + R1C1R2C2d2vo/dt2

or    d2vo/dt2 + [(1/(R1C1)) + (1/(R2C2))]dvo/dt + vo/(R1R2C1C2)  =  vs/(R1R2C1C2)

With the given parameters,

(R1R2C1C2)  =  104x104x20x10-6x100x10-6  =  2x10-2

1/(R1R2C1C2)  =  5

[(1/(R1C1)) + (1/(R2C2))]  =  10-4[(1/20x10-6) + (1/200x10-6)]  =  6

Hence,  we now have s2 + 6s + 5  =  0  =  (s +1)(s + 5)

Therefore von  =  Ae-t + Be-5t,  and  vof  =  10V

Thus,				vo  =  10 + Ae-t + Be-5t				(5)

For  t < 0,  vs  =  0,  v1(0–)  =  0  =  vo(0–)

For  t > 0,  vs  =  10,  but

v1(0+)  – vo(0+)  =0				(6)

From (2),		dvo(0+)/dt  =  [v1(0+) – vo(0+)]/R2C2  =  0		(7)

Imposing these conditions on vo(t),

0  =  10 + A + B				(8)

0  =  –A – 5B  or  A  =  –5B			(9)

From (8) and (9),  A  =  –12.5  and  B  =  2.5

vo(t)  =  (10 – 12.5e-t + 2.5e-5t) V,  t > 0

P.P.8.12		We follow the same procedure as in Example 8.12.  The schematic is shown in Figure (a).  The current marker is inserted to display the inductor current.  After simulating the circuit, the required inductor current is plotted in Figure (b).

#### (a)

P.P.8.13		When the switch is at position a, the schematic is as shown in Figure (a).  We carry out dc analysis on this to obtain initial conditions.  It is evident that vC(0)  =  8 volts.

With the switch in position b, the schematic is as shown in Figure (b).  A voltage marker is inserted to display the capacitor voltage.  When the schematic is saved and run, the output is as shown in Figure (c).

#### (b)

P.P.8.14		The dual circuit is obtained from the original circuit as shown in Figure (a).  It is redrawn as shown in Figure (b).

#### 0.1

### 10

#### (a)

#### 3 H

#### 4 H

#### 3 F

#### 4 F

##### 50mV

+

#### 50mA

#### 0.1

#### (b)

#### 3 H

#### 4 F

##### 50mV

+

P.P.8.15		The dual circuit is obtained in Figure (a) and redrawn in Figure (b).

#### 5

### 3

#### (a)

#### 4 H

#### 0.2 H

#### 4 F

#### 0.2F

##### 20 V

+

#### 2A

#### 20A

##### 2V

+

### 0.2

#### 1/3

#### (b)

#### 0.2 H

#### 4F

#### 20A

##### 2V

+

### 1/5

#### 1/3

# P.P.8.16		Since  12  =  4i + vL + vC    or   vC  =  12 – 4i - vL

–(vC – 12)  =  4i + vL  =  e-250t(12cosdt + 0.2684sindt – 268sindt)

vC(t)  =  12 – 12e-250t cos 11,180t + 267.7e-250t sin 11,180t V

P.P.8.17		We follow the same procedure as in Example 8.17.  The schematic is as shown in Figure (a) with two voltage markers to display both input and output voltages.  When the schematic is saved and run, the result is as displayed in Figure (b).

#### (a)

#### (a)