# Chapter 2, Solution 39

Evaluate Req for each of the circuits shown in Fig. 2.103.

4 k

6 k

12 k

12 k

(b)

2 k

2 k

1 k

1 k

(a)

Figure 2.103

For Prob. 2.39.

Solution

(a)   We note that the top 2k-ohm resistor is actually in parallel with the first 1k-ohm resistor.  This can be replaced (2/3)k-ohm resistor.  This is now in series with the second 2k-ohm resistor which produces a 2.667k-ohm resistor which is now in parallel with the second 1k-ohm resistor.  This now leads to,

Req = [(1x2.667)/3.667]k = 727.3 Ω.

(b)   We note that the two 12k-ohm resistors are in parallel producing a 6k-ohm resistor.  This is in series with the 6k-ohm resistor which results in a 12k-ohm resistor which is in parallel with the 4k-ohm resistor producing,

Req = [(4x12)/16]k = 3 kΩ.