Chapter 3, Solution 15

#### io

+

##### 10 V

#### 3 S

### 6 S

### 4 A

#### 2 A

### 5 S

Apply nodal analysis to find io and the power dissipated in each resistor in the circuit of Fig. 3.64.

Figure 3.64

For Prob. 3.15.

Solution

### V1

### V2

#### io

+

##### 10 V

#### 3 S

### 6 S

### 4 A

#### 2 A

### 5 S

Step 1.		First pick a reference node and write the appropriate node equations.  In addition, we will need a constraint equation to solve for io.  Then solve for the node voltages.

[6(V1–0)] + 2 + [5(V1–10–0)] + [3(V1–10–V2)] = 0;  [3(V2–(V1–10))] – 2 – 4 = 0; and

io = 6(V1–0).

Step 2.		(6+5+3)V1 – 3V2 = –2 + 50 + 30 or 14V1 – 3V2 = 78 and –3V1 + 3V2 = –24.

14-3-33V1V2=78-24or V1V2=3331442-978-24

V1 = 3(78–24)/33 = 4.909 V and V2 = [(3x78)–(14x24)]/33 = –3.091 V.  Let I3 = 3(4.909–10–(–3.091)) = –6 A and I5 = 5(4.909–10) = –25.455 A.  Finally, io = 6x4.909

= 29.45 A.  This now gives us, P6S = (29.454)2/6 = 144.6 W;

P5S = (–25.455)2/5 = 129.6 W; and P3S = (–6)2/3 = 12 W.