Chapter 3, Solution 92

Using Fig. 3.28, design a problem to help other students better understand transistors.  Make sure you use reasonable numbers!

R1

R2

+

V1

–

R3

Figure 3.128

For Prob. 3.92.

Solution

Although there are many ways to work this problem, this is an example based on the same kind of problem asked in the third edition.

Problem

Find IB and VC for the circuit in Fig. 3.128. Let  = 100, VBE = 0.7V.

Solution

+

V0

–

# 4 k

-

+

# 12V

+

VCE

–

+

VBE

–

# IB

# IE

# 10 k

# 5 k

# VC

# IC

# I1

I1 = IB + IC = (1 + )IB  and  IE = IB + IC = I1

Applying KVL around the outer loop,

4kIE  + VBE + 10kIB + 5kI1 = 12

12 – 0.7 = 5k(1 + )IB + 10kIB + 4k(1 + )IB = 919kIB

IB = 11.3/919k = 12.296 A

Also,	12 = 5kI1 + VC  which leads to  VC = 12 – 5k(101)IB = 5.791 volts