Chapter 4, Problem 83.

The following results were obtained from measurements taken between the two terminals of a resistive network.

| Terminal Voltage | 12 V | 0 V |
| --- | --- | --- |
| Terminal Current | 0 V | 1.5A |

Find the Thevenin equivalent of the network.