Chapter 4, Problem 95.

A dc voltmeter with a sensitivity of 20 k/V is used to find the Thevenin equivalent of a linear network. Readings on two scales are as follows:

- 0-10 V scale: 4 V

- 0-50 V scale: 5 V

Obtain the Thevenin voltage and the Thevenin resistance of the network.