Chapter 4, Solution 45.

Find the Thevenin equivalent of the circuit in Fig. 4.112 as seen by looking into terminals a and b.

Figure 4.112

For Prob. 4.45.

Solution

For RN, consider the circuit in Fig. (a).

### (b)

### 4

#### 6

### 6

### IN

##### 4A

### (a)

### 4

#### 6

### 6

### RN

RN  =  (6 + 6)||4  =  3 Ω

For IN, consider the circuit in Fig. (b).  The 4-ohm resistor is shorted so that 4-A current is equally divided between the two 6-ohm resistors.  Hence,

IN  =  4/2  =  2 A and

VThev = 2x3 = 6 V