Chapter 4, Solution 52.

For the transistor model in Fig. 4.118, obtain the Thevenin equivalent at terminals a-b.

12 V

Figure 4.118

For Prob. 4.52.

Solution

Step 1.		To find the Thevenin equivalent for this circuit we need to find voc and isc.

+

20Io

voc

2 k

#### b

#### a

#### Io

12V

+

3 k

–

isc

Then VThev = voc and Req = voc/isc.

For voc, Io = (12–0)/3k = 4 mA and 20Io + (voc–0)/2k = 0.

For isc, isc = –20Io.

Step 2.		voc = –2k(20Io) = –40x4 = –160 volts = VThev

isc = –20x4x10–3 = –80 mA or

Req = –160/(80x10–3) = 2 kΩ.