Chapter 4, Solution 66.

Find the maximum power that can be delivered to the resistor R in the circuit in Fig. 4.132.

Figure 4.132

For Prob. 4.66.

Solution

#### 3

### 5

#### (a)

#### b

#### a

##### RTh

#### 2

#### i

##### 20V

+

+

#### 3

### 5

#### (b)

#### b

#### a

##### VTh

#### 2

+

+

##### 30V

#### 10V

We first find the Thevenin equivalent at terminals a and b.  We find RTh using the circuit in Fig. (a).

RTh  =  2||(3 + 5)  =  2||8  =  1.6 ohms

By performing source transformation on the given circuit, we obatin the circuit in (b).  We now use this to find VTh.

10i + 30 + 20 + 10  =  0,  or  i  =  –6

VTh + 10 + 2i  =  0,  or  VTh  =  2 V

p  =  VTh2/(4RTh)  =  (2)2/[4(1.6)]  =  625 m watts