Chapter 4, Solution 75.

For the circuit in Fig. 4.141, determine the value of R such that the maximum power delivered to the load is 3 mW.

Figure 4.141

For Prob. 4.75.

Solution

# We need to first find RTh and VTh.

#### vo

##### +

##### VTh

#### (b)

##### 3V

+

#### R

#### R

#### R

##### 2V

+

##### 1V

+

### R

#### (a)

##### RTh

### R

### R

Consider the circuit in Fig. (a).

(1/Req)  =  (1/R) + (1/R) + (1/R)  =  3/R

Req  =  R/3

From the circuit in Fig. (b),

((1 – vo)/R) + ((2 – vo)/R) + ((3 – vo)/R)  =  0

vo  =  2  =  VTh

For maximum power transfer,

RL  =  RTh  =   R/3

Pmax  =  [(VTh)2/(4RTh)]  =  3 mW

RTh =  [(VTh)2/(4Pmax)]  =  4/(4xPmax)  =  1/Pmax =  R/3

R  =  3/(3x10-3)  =  1 kΩ

1 kΩ, 3 mW