Chapter 4, Solution 83.

The following results were obtained from measurements taken between the two terminals of a resistive network.

| Terminal Voltage | 12 V | 0 V |
| --- | --- | --- |
| Terminal Current | 0 V | 1.5A |

Find the Thevenin equivalent of the network.

Solution

VTh  =  Voc  =  12 V,  Isc  =  IN  =  1.5 A

RTh  =  VTh/IN  =  8 ohms,  VTh  =  12 V,  RTh  =  8 ohms