Chapter 4, Solution 86.

A black box with a circuit in it is connected to a variable resistor. An ideal ammeter (with zero resistance) and an ideal voltmeter (with infinite resistance) are used to measure current and voltage as shown in Fig. 4.143. The results are shown in the table below.

Figure 4.143

For Prob. 4.86.

- Find i when R = 4 .

- Determine the maximum power from the box.

| R() | V(V) | i(A) |
| --- | --- | --- |
| 2 | 3 | 1.5 |
| 8 | 8 | 1.0 |
| 14 | 10.5 | 0.75 |

Solution

We replace the box with the Thevenin equivalent.

#### RTh

##### VTh

+

### R

#### i

##### +

##### v

VTh  =  v + iRTh

When i  =  1.5,  v  =  3,  which implies that VTh  =  3 + 1.5RTh		(1)

When i  =  1,  v  =  8, which implies that VTh  =  8 + 1xRTh			(2)

From (1) and (2),  RTh  =  10 ohms and VTh  = 18 V.

(a)	When R  =  4, i  =  VTh/(R + RTh)  =  18/(4 + 10)  =  1.2857 A

(b)	For maximum power, R  =  RTH

Pmax  =  (VTh)2/4RTh  =  182/(4x10)  =  8.1 watts