Chapter 4, Solution 91.

- In the Wheatstone bridge circuit of Fig. 4.147 select the values of R1 and R3 such that the bridge can measure Rx in the range of 0-10 .

- Repeat for the range of 0-100 .

Figure 4.147

For Prob. 4.91.

Solution

Rx  =  (R3/R1)R2

(a)	Since 0 < R2 < 50 ohms, to make 0 < Rx < 10 ohms requires that when R2  =  50 ohms, Rx  = 10 ohms.

10  =  (R3/R1)50  or  R3  =  R1/5

so we select R1  =  100 ohms  and  R3  =  20 ohms

(b)	For 0 < Rx < 100 ohms

100  =  (R3/R1)50,  or  R3  =  2R1

So we can select R1  =  100 ohms  and   R3  =  200 ohms